# Robotics Math Roadmap for Numericals

Based on your exam pattern (Total Marks 75: Part A-MCQ 15 marks, Part B Descriptive and solving problems 60 marks) and syllabus (Niku & Pratihar), here is a neatly formatted, sequential roadmap covering all important formulas and related theory.

## Phase 1: The Foundation

Topic: Degrees of Freedom (DOF)

Understand the basic mobility of a mechanism.

- Theory: DOF is typically determined by the number of joint connectivities, except in closed-loop parallel manipulators. A planar arm link in space has 3 DOF, while a spatial link has 6 DOF.

- Formula (Grübler’s Criterion for Planar Mechanisms): M = 3(n - 1) - 2j (where n is the number of links including the base, and j is the number of 1-DOF joints).

- Formula (General Spatial): M = 6n – Σm(6 – ci)

Topic: Spatial Transformations

Representing position and orientation in 3D space.

- Theory: We use a 4x4 Homogeneous Transformation Matrix (SE(3)) to define a frame's position and orientation relative to a reference frame. This matrix combines a 3x3 Rotation matrix (SO(3)) and a 3x1 translation vector.

- Structure:

T = [ R  | P ]
    [ 0  | 1 ]

- Rules of Compound Transformations:

  - If rotation/translation is with reference to the Universal (fixed) Frame: PRE-MULTIPLY.

  - If rotation/translation is with reference to the Moving (Tool) Frame: POST-MULTIPLY.

- Inverse Matrix Shortcut:

T⁻¹ = [ R^T  | -R^T * P ]
      [ 0    |    1     ]

## Phase 2: Forward Kinematics (FK)

Topic: Denavit-Hartenberg (D-H) Notations

A systematic way to assign coordinate frames and find the transformation matrix of a robot.

- Theory: The D-H convention uses four parameters to describe the transformation from link i-1 to link i:

  - θᵢ (Joint Angle): Rotation about Zᵢ₋₁ to make Xᵢ₋₁ parallel to Xᵢ.

  - dᵢ (Link Offset): Translation along Zᵢ₋₁ to make Xᵢ₋₁ collinear with Xᵢ.

  - aᵢ (Link Length): Translation along Xᵢ to align the origins.

  - αᵢ (Link Twist): Rotation about Xᵢ to align Zᵢ₋₁ with Zᵢ.

- Formula (Standard D-H Transformation Matrix Aᵢ):

Aᵢ = Rot(Z, θᵢ) * Trans(0, 0, dᵢ) * Trans(aᵢ, 0, 0) * Rot(X, αᵢ)
Aᵢ = [ cos(θᵢ)  -sin(θᵢ)cos(αᵢ)   sin(θᵢ)sin(αᵢ)   aᵢ*cos(θᵢ) ]
     [ sin(θᵢ)   cos(θᵢ)cos(αᵢ)  -cos(θᵢ)sin(αᵢ)   aᵢ*sin(θᵢ) ]
     [   0         sin(αᵢ)           cos(αᵢ)          dᵢ      ]
     [   0            0                 0              1      ]

- Total Transformation: ⁰Tₙ = A₁ * A₂ * ... * Aₙ

## Phase 3: Inverse Kinematics (IK)

Theory: Solving for the joint variables (angles or distances) given the desired end-effector position and orientation. This is generally more complex than FK and may have multiple solutions, no solutions, or singularities.

Approach (Algebraic): Equate the elements of the given target transformation matrix to the symbolic elements of your computed ⁰Tₙ matrix and solve for the variables (often using atan2 for robust angle finding).

## Phase 4: Velocity Analysis & Manipulability

Topic: The Jacobian Matrix

Relating joint velocities to end-effector velocities.

- Theory: The Jacobian (J) maps velocities in joint space to velocities in Cartesian workspace (V = J * q_dot). It is also used to map static forces (τ = J^T * F).

- Formula: Take the partial derivatives of the forward kinematic position equations (x, y, z) with respect to each joint variable (q₁, q₂, etc.).

J = [ ∂x/∂q₁  ∂x/∂q₂  ... ]
    [ ∂y/∂q₁  ∂y/∂q₂  ... ]
    [ ∂z/∂q₁  ∂z/∂q₂  ... ]

Topic: Manipulability & Singularities

- Theory: A singularity occurs when a robot loses one or more degrees of freedom (it gets "stuck" in certain directions). At a singularity, the determinant of the Jacobian is zero.

- Yoshikawa’s Manipulability Measure (w): w = sqrt(det(J * J^T)). If the robot is not redundant (square Jacobian), this simplifies to w = |det(J)|. Higher values indicate better dexterity.

## Phase 5: Trajectory Planning

Theory: Planning how the robot moves from Point A to Point B smoothly over time, avoiding jerky motions that damage actuators.

- Cubic Polynomial Trajectory: Typically uses 4 boundary conditions to find coefficients (a₀, a₁, a₂, a₃):

  - Initial position (t=0)

  - Final position (t=tf)

  - Initial velocity = 0

  - Final velocity = 0

## Phase 6: Robot Motion Planning

Theory: How a robot navigates an environment to reach a goal without hitting obstacles.

- Configuration Space (C-Space): Representing the robot as a single point and expanding the obstacles by the robot's radius.

- Graph Methods:

  - Visibility Graph: Connects vertices of obstacles to find shortest paths.

  - Voronoi Diagram: Paths stay as far away from obstacles as possible (safer, but longer).

  - Cell Decomposition: Breaking the free space into smaller regions (cells).

- Constraints:

  - Holonomic: Controllable DOF = Total DOF (can move instantly in any direction, like a quadrotor).

  - Non-holonomic: Controllable DOF < Total DOF (movement is restricted, like a car needing to parallel park).

## Bonus: Aerial Robotics (Drones)

- Quadrotors (Rotary Wing): 6 DOF but only 4 actuators (underactuated).

  - Hover: All 4 rotors spin at the same speed.

  - Pitch/Roll: Differential thrust between front/back or left/right rotors.

  - Yaw: Differential torque between the clockwise pair and counter-clockwise pair of rotors.

- Fixed Wing: Uses control surfaces.

  - Ailerons: Control Roll.

  - Elevators: Control Pitch.

  - Rudder: Controls Yaw.